Exp-4 MC

MATLAB code:

All in one code: (for 2 graphs directly)

clear all

clc

N = 10^6; % number of bits or symbols rand('state',100);

% initializing the rand() function randn('state',200);

% initializing the randn() function

% Transmitter

ip = rand(1,N)>0.5; % generating 0,1 with equal probability

s = 2*ip-1; % BPSK modulation 0 -> -1; 1 -> 1

n = 1/sqrt(2)*[randn(1,N) + j*randn(1,N)]; % white gaussian noise, 0dB variance

Eb_N0_dB = [-3:10]; % multiple Eb/N0 values

for ii = 1:length(Eb_N0_dB)

% noise addition

y = s + 10^(-Eb_N0_dB(ii)/20)*n; % additive white gaussian noise

% receiver - hard decision decoding

ipHat = real(y)>0;

% counting the errors

nErr(ii) = size(find([ip - ipHat]),2);

end

simBer = nErr/N; % simulated ber

theoryBer = 0.5*erfc(sqrt(10.^(Eb_N0_dB/10))); % theoretical ber

% plot

close all

figure

semilogy(Eb_N0_dB,theoryBer,'b.-');

hold on

semilogy(Eb_N0_dB,simBer,'mx-');

axis([-3 10 10^-5 0.5])

grid on

legend('theory','simulation');

xlabel('Eb/No, dB');

ylabel('Bit Error Rate');

title('Bit error probability curve for BPSK modulation');

clear all

clc

N = 10^6; % Number of bits

% Generate random bits

ip = rand(1,N) > 0.5;

% BPSK modulation

s = 2*ip - 1;

EbNodB = 0:35;

for ii = 1:length(EbNodB)

EbNo = 10^(EbNodB(ii)/10);

% Rayleigh fading channel

h = (1/sqrt(2)) * (randn(1,N) + 1j*randn(1,N));

% AWGN noise

n = (1/sqrt(2)) * (randn(1,N) + 1j*randn(1,N));

% Received signal

y = h.*s + (10^(-EbNodB(ii)/20))*n;

% Equalization

yHat = y ./ h;

% Decision device

ipHat = real(yHat) > 0;

% Count errors

nErr(ii) = sum(ip ~= ipHat);

end

% Simulated BER

simBer = nErr / N;

% Theoretical BER for Rayleigh channel

EbNoLin = 10.^(EbNodB/10);

theoryBerRayleigh = 0.5 * (1 - sqrt(EbNoLin ./ (1 + EbNoLin)));

% Theoretical BER for AWGN channel

theoryBerAWGN = 0.5 * erfc(sqrt(EbNoLin));

%% Plotting

figure

semilogy(EbNodB, theoryBerAWGN,'b','LineWidth',2)

hold on

semilogy(EbNodB, theoryBerRayleigh,'r-s','LineWidth',2)

semilogy(EbNodB, simBer,'g-x','LineWidth',2)

grid on

xlabel('Eb/No, dB')

ylabel('Bit Error Rate')

title('BER for BPSK modulation in Rayleigh channel')

legend('AWGN-Theory','Rayleigh-Theory','Rayleigh-Simulation')

clear;

N = 10^6;                          % number of bits

% Transmitter

ip = rand(1,N) > 0.5;              % generate 0 or 1

s = 2*ip - 1;                      % BPSK modulation: 0 -> -1, 1 -> +1

% Eb/N0 range in dB

Eb_N0_dB = 0:1:35;

simBer = zeros(size(Eb_N0_dB));

for ii = 1:length(Eb_N0_dB)

% Rayleigh fading channel (complex Gaussian)

h = 1/sqrt(2)*(randn(1,N) + 1j*randn(1,N));

% AWGN noise

n = 1/sqrt(2)*(randn(1,N) + 1j*randn(1,N));

% Received signal: faded + noisy

y = h.*s + 10^(-Eb_N0_dB(ii)/20)*n;

% Equalization

yHat = y ./ h;

% Receiver

ipHat = real(yHat) > 0;

% BER calculation

simBer(ii) = sum(ip ~= ipHat)/N;

end

% Theoretical BER (AWGN and Rayleigh)

EbNO = 10.^(Eb_N0_dB/10);

theoryBer_AWGN = 0.5 * erfc(sqrt(EbNO));

theoryBer_Rayleigh = 0.5 * (1 - sqrt(EbNO ./ (1 + EbNO)));

% Plot

semilogy(Eb_N0_dB, theoryBer_AWGN, 'c*-', ...

Eb_N0_dB, theoryBer_Rayleigh, 'bs-', ...

Eb_N0_dB, simBer, 'mx-');

grid on;

xlabel('Eb/No, dB');

ylabel('Bit Error Rate');

title('BER for BPSK modulation in Rayleigh channel');

legend('AWGN-Theory', 'Rayleigh-Theory', 'Rayleigh-Simulation');

axis([0 35 1e-5 1]);

1.  Bit error probability curve for BPSK modulation

clear all

clc

N = 10^6; % number of bits or symbols

rand('state',100);

randn('state',200);

% Transmitter

ip = rand(1,N) > 0.5;

% BPSK Modulation

s = 2*ip - 1;

% AWGN Noise

n = 1/sqrt(2) * (randn(1,N) + 1j*randn(1,N));

Eb_N0_dB = -3:10;

for ii = 1:length(Eb_N0_dB)

% Noise addition

y = s + 10^(-Eb_N0_dB(ii)/20) * n;

% Receiver

ipHat = real(y) > 0;

% Error calculation

nErr(ii) = sum(ip ~= ipHat);

end

% Simulated BER

simBer = nErr / N;

% Theoretical BER

theoryBer = 0.5 * erfc(sqrt(10.^(Eb_N0_dB/10)));

% Plot

figure

semilogy(Eb_N0_dB, theoryBer, 'b.-', 'LineWidth',2);

hold on

semilogy(Eb_N0_dB, simBer, 'mx-', 'LineWidth',2);

axis([-3 10 10^-5 0.5])

grid on

legend('Theory','Simulation');

xlabel('Eb/No (dB)');

ylabel('Bit Error Rate');

title('BER Curve for BPSK over AWGN Channel');

2. BER for BPSK modulation in Rayleigh channel

clc;

clear;

close all;

% Number of bits

N = 1e6;

% Eb/No range in dB

EbN0_dB = 0:1:35;

EbN0 = 10.^(EbN0_dB/10);

% Theoretical BER for AWGN (BPSK)

ber_awgn = 0.5 * erfc(sqrt(EbN0));

% Theoretical BER for Rayleigh fading

ber_rayleigh_theory = 0.5 * (1 - sqrt(EbN0 ./ (1 + EbN0)));

% Initialize simulation BER

ber_rayleigh_sim = zeros(size(EbN0_dB));

for k = 1:length(EbN0_dB)

% Generate random bits

bits = randi([0 1], N, 1);

% BPSK modulation

tx = 2*bits - 1;

% Rayleigh fading channel

h = (randn(N,1) + 1i*randn(N,1)) / sqrt(2);

% Noise variance

noise_sigma = sqrt(1/(2*EbN0(k)));

% AWGN noise

noise = noise_sigma * (randn(N,1) + 1i*randn(N,1));

% Received signal

rx = h .* tx + noise;

% Equalization

rx_eq = rx ./ h;

% Detection

bits_hat = real(rx_eq) > 0;

% BER calculation

ber_rayleigh_sim(k) = sum(bits ~= bits_hat) / N;

end

% Plot

figure;

semilogy(EbN0_dB, ber_awgn, 'b-+', 'LineWidth', 1.5);

hold on;

semilogy(EbN0_dB, ber_rayleigh_theory, 'r-s', 'LineWidth', 1.5);

semilogy(EbN0_dB, ber_rayleigh_sim, 'g-*', 'LineWidth', 1.5);

grid on;

xlabel('Eb/No, dB');

ylabel('Bit Error Rate');

title('BER for BPSK modulation in Rayleigh channel');

legend('AWGN-Theory', 'Rayleigh-Theory', 'Rayleigh-Simulation');

axis([0 35 1e-5 1]);

set(gca, 'YScale', 'log');